## MORE DETAILED REPORT ON THE FEASIBILITY STUDY

1D   EXISTING POOL REFURBISHMENT OPTION B (Refurbished pool, Heated Pool, New Enclosed Building)

The proposed works may be summarised as follows:

•           As Option 1B with the addition of a New Enclosed Building.

The option includes for a new building covering the minimum required area. The building is detailed to building regulation standard, in terms of general construction, insulation and ventilation.

Because of the nature of the facility, a heated swimming pool, the building is required to have sufficient ventilation and insulation in order to provide a suitable working atmosphere and to alleviate condensation. This requires that a full mechanical ventilation and heated air system be constructed along with the relatively high specification building fabric.

The building fabric proposed is as economical a possible and will have the general appearance of an industrial unit built alongside and immediately adjacent the existing dressing room block. However the building has high insulation levels. It is proposed that the building will be constructed of a portal frame system with in-filled insulated concrete block walls to approx. 3m high. The upper approx. 2m along with a shallow roof pitch on purlin supports will be clad with insulated cladding panels.

The portal frame is proposed as a laminated timber truss system, which is a common swimming pool feature. The cost estimate of this system is deemed to be similar to a steel frame system. The timber frame is more suitable in the long term for the atmosphere and conditions of the internal heated pool, and has a level of aesthetic suitability which helps to alleviate the required industrial nature of the remainder of the facility.

KIDS POOL OPTIONS

The inclusion of the kids pool must include the extension of the grounds, the building over, refurbishment of the pool and deck,  filtration, heating of the pool which may be a separate system, walls, lighting etc. For this reason the feasibility study prioritises the most economical options for redevelopment of the existing facilities as being without the inclusion of the kids pool.